# Sable Ridge Tunnel Gas Safety Procedure

This procedure applies to every shift working in the Sable Ridge haulage tunnel.

# Gas monitoring

Methane is measured at the tunnel face at the start of each shift and every two hours.

If methane reaches 1.0%, stop hot work and repeat the measurement after five minutes.

If methane reaches 1.5% or more, shut down all machinery immediately and evacuate the face to the refuge station.

# Exposure limits

| Gas | Action level | Evacuation level |
| --- | --- | --- |
| Methane | 1.0% | 1.5% |
| Carbon monoxide | 30 ppm | 100 ppm |
| Hydrogen sulphide | 5 ppm | 10 ppm |

# Responsibilities

The shift supervisor records every reading in the gas log.

The ventilation officer checks the main fan daily.

Every worker carries a personal gas detector.

# Re-entry

After an evacuation, nobody re-enters the face until two consecutive readings, taken ten minutes apart, are below the action level.